Applications of SIX SIGMA in Industries of Manufacturing

Final Project Report

ISE 250- Leading Six Sigma Improvement

Prof. Dr Jacob Tsao

San Jose State University

Engineering Management

Group 18

Sri Soumya Nallagatla, Geethan Rajkumar, Dinesh Satya Harsha Koya

May 11, 2023

Table of Contents

Abstract	3

Introduction	4

Study Plan	5

Methodologies	6

Case Study 1: Horn System Assembly Improvement in the Automotive Industry	7

Case Study 2: Nutritional Production Efficiency Enhancement	8

Case Study 3: Steel Pipe Manufacturing Waste Reduction	9

Synergy and Criticism	10

Conclusion	11

References……………………………………………………………………………………11

Abstract

This extensive paper looks at three manufacturing sectors—automotive assembly, nutritional production, and steel pipe manufacturing—and how each applies the DMAIC (Define, Measure, Analyse, Improve, Control) technique. Every case study is carefully examined to show how DMAIC, a fundamental component of Six Sigma concepts, is skilfully utilised to solve certain problems that are unique to a given industry. The results from each industry show significant gains in product quality, sustainability, and operational efficiency.

DMAIC implementation significantly reduced assembly line faults in the automobile industry, improving product quality and customer satisfaction. DMAIC was applied in the nutritional business to maximize packing line efficiency, which decreased operating costs and raised manufacturing output. In the meanwhile, DMAIC enabled a significant reduction in production waste in the steel pipe industry, supporting initiatives for cost containment and environmental sustainability.

This study emphasizes DMAIC's flexibility and efficacy in a range of industrial settings, as well as the methodology's strategic importance in fostering notable advancements. The examined organizations met and frequently surpassed their objectives for operational improvement by following a systematic execution of each DMAIC phase that was specially designed to meet the demands of their respective industries. The case examples exhibit synergies that bolster the adaptability of DMAIC and highlight its potential to promote operational excellence and continuous improvement in a range of industrial areas.

Through a detailed examination of the implementation of DMAIC and the resulting consequences, this paper offers significant insights into the transformational potential of structured problem-solving and quality improvement approaches in manufacturing. It is a useful tool for businesses looking to use DMAIC to improve productivity, guarantee quality, and gain a competitive edge in the manufacturing industry.

Introduction

A data-driven improvement cycle called DMAIC (Define, Measure, Analyse, Improve, Control) is used to optimise and stabilise business processes and designs. DMAIC, which is based on Six Sigma concepts, offers an organised method for solving problems that enables businesses to identify their pain spots, cut down on mistakes, and enhance quality across a range of operational aspects. For industries looking to reduce errors, boost output, and guarantee consistent results in a market that is getting more and more competitive, this technique is essential.

The usefulness and flexibility of DMAIC are demonstrated by its application into various production contexts. (Pugna et al., 2016)DMAIC has shown to be crucial in assisting companies in achieving notable operational gains, whether they are optimising waste reduction in the steel pipe manufacturing process, enhancing nutritional production efficiency, or optimising automobile assembly lines. (Krotov et al., 2017)This is especially important in industries where productivity and accuracy have a direct impact on customer happiness and financial performance. Because of its universality, the approach may be used in a variety of industries, tailoring its phases to individual industrial requirements while always pushing the limits of process optimisation.

Every stage of the DMAIC has a strategic purpose: (Saleh et al., 2023)Clearly defines the issue and establishes the objectives for change; Measure describes the condition of the process now and pinpoints areas that need to be changed using data; Analyse aids in determining the underlying causes of flaws or inefficiency; Enhance concentrates on finding answers to these issues and putting them into practice; additionally Control seeks to guarantee that the gains are long-lasting and to preserve the efficacy of the solution throughout time. This methodical technique guarantees that enhancements are not only executed but also enduring, ultimately contributing to the organization's sustained prosperity.

In this research, we examine the DMAIC methodology's use in three particular case studies from the steel pipe, nutritional, and automobile production sectors. (Krotov et al., 2017)Every scenario offers distinct difficulties and demonstrates how DMAIC was applied to address particular operational problems, resulting in quantifiable gains. Through an analysis of these examples, the paper seeks to show how DMAIC is a reliable instrument for operational excellence and quality management, and how it may revolutionise business processes across a range of industries. Our goal in doing this analysis is to offer practical suggestions and a more profound comprehension of how DMAIC concepts may be efficiently customised to improve process effectiveness, sustainability, and quality in various manufacturing domains.

# Study Plan

This study's main goal is to thoroughly examine how the DMAIC (Define, Measure, Analyse, Improve, Control) approach is applied in three different manufacturing sectors: steel pipe manufacture, nutritional production, and automobile assembly. The study was carefully designed to offer an in-depth investigation of how DMAIC principles are customised to suit certain industry-specific difficulties, emphasising its efficacy in raising operating efficiency, cutting waste, and increasing quality.

The study was designed to accomplish these goals by carefully analysing three case studies. Every case was chosen with care to showcase the DMAIC approach's adaptability to diverse operating situations and to represent a distinct industry. These instances serve as excellent examples of how DMAIC may be applied in a variety of contexts as extensive data was available, the DMAIC process was clearly documented, and noteworthy results were reported.

The study's approach comprised a thorough analysis of each DMAIC step as it was used in the selected case studies. The following factors were examined in each case:

- Define Phase: Problem, project scope, and goals are identified.

- Measure Phase: Techniques for gathering data and preliminary results utilised to determine performance baselines.

- Analyse Phase: Methods and resources for discovering the core reasons of inefficiency or problems with quality.

- Improve Phase: Root cause analysis strategies and solutions are put into practice.

- Control Phase: Systems implemented to maintain the gains made.

By contrasting methods, resources, and results amongst the examples, the study sought to draw practical conclusions and lessons from each implementation. This methodology not only illustrates DMAIC's flexibility, but also its effect on industry-specific problems. In addition to provide a benchmark for companies looking to implement DMAIC for process improvement, the research aims to add to the body of literature by shedding light on the application of Six Sigma methodology in the manufacturing sector.

Methodologies

The methods applied in the three case studies centred on DMAIC principles offer an organised strategy for addressing issues unique to a certain sector. The specific tools and methods used in each DMAIC phase—Define, Measure, Analyse, Improve, and Control—are suited to the particular requirements of the steel pipe, nutritional, and automobile assembly industries.

- Define: In all three studies, this stage entailed precisely describing and characterising the issue as well as establishing quantifiable objectives. For example, in nutritional production, the emphasis was on improving line efficiency, whereas in automobile assembly, the goal was to lower failure rates. Process mapping was aided by tools such as the SIPOC (Suppliers, Inputs, Process, Outputs, Customers) diagrams, which established the framework for focused enhancements.

- Measure: The goal of this stage was to gather pertinent information in order to provide baselines for comparisons later on. Statistics on the frequency of defects were obtained in the automobile sector. Metrics pertaining to packing line downtimes were documented in the nutritional business, while waste levels were measured in the steel pipe sector. Statistical software for data analysis was a common component of measurement equipment, giving a comprehensive image of the process capabilities and problem areas.

- Analyse: To find the underlying reasons of inefficiencies, the analysis entailed going deeply into the data that had been gathered. Methods like Pareto charts and fishbone diagrams were frequently employed in the research to identify primary sources of errors or inefficiencies, allowing for a targeted approach to problem-solving.

- Improve: During this stage, strategies were created and put into practice to deal with the problems that were found. For instance, operator training and machine calibration were improved in the car assembly industry. Automation was brought into the nutritional industry to simplify procedures and minimise human error. To reduce waste in the production of steel pipes, equipment modifications and process redesigns were used.

- Control: The last stage made sure that the advancements would last in the long run. This required putting in place control measures including frequent audits and continuous performance monitoring. In order to sustain process improvements and guarantee ongoing compliance with the new standards, dashboards and real-time data tracking were often used.

Overall, each industry was able to use the DMAIC Framework to successfully solve its unique operational difficulties since these techniques, although conventional in their approach, were customized in their application.

Case Study 1: Horn System Assembly Improvement in the Automotive Industry

DEFINE: What is the quality problem?

The wrong height of rivets utilised during the assembly process was found to be the main quality fault in the horn system production line employed by the automobile industry. Due to this issue, the horn systems' dependability was being severely impacted, which increased the semi-finished product failure rate. The entire quality of the car was impacted by the uneven rivet heights, which jeopardised the horn systems' functionality and structural integrity.

MEASURE: What are the measures?
To assess and solve the quality issue efficiently, "Rivet Height" was determined to be the important measure. (Pugna et al., 2016)A statistically significant set of samples had the height of each rivet measured with precision instruments like callipers. In order to obtain precise and trustworthy data and provide a clear picture of the scope and variability of the problem across several manufacturing batches, this measuring procedure was crucial.

Analyse: What is the root cause? How was it found?
A root cause analysis was performed in order to identify the underlying causes of the erroneous rivet height. (Pugna et al., 2016)An Ishikawa diagram and the "5 Whys" technique allowed for a methodical investigation of possible reasons, which identified operator weariness and incorrect riveting force as the main culprits. This research, which concentrated on procedural mistakes rather than material flaws, was essential in tracking the deficiencies back to their sources.

IMPROVE: What was the solution?
The root cause analysis was used to propose and execute a number of modifications. A softer release spring was incorporated into the riveting hand tool's redesign, which assisted in delivering the proper force throughout the procedure and guaranteed uniform rivet height. In order to avoid mistakes, a Poka-Yoke device was also added. If the wrong force was used, it would notify or halt the operation. By addressing the underlying problems that were found, these treatments sought to standardise the riveting procedure and lessen the physical strain on operators.

Control: How can we make sure we keep winning? 
Ongoing monitoring and control procedures were put in place to preserve the advancements and guarantee that the failure rates stayed low. This involved using control charts to continually track the rivet height and conducting routine audits. These instruments aided in promptly detecting any departures from the established norms, enabling prompt remedial measures. To ensure the long-term effectiveness of the improvement measures, additional modifications and ongoing training for operators were planned to react to any new issues that could develop.

Discussion on Case's Accomplishments and Improvement Idea:

By implementing focused enhancements, the failure rate of horn systems was significantly decreased, improving product quality and lowering rework expenses. In addition to resolving the particular problem of rivet height, the proactive strategy increased process efficiency in general. One other suggestion for improvement would be to include real-time data analytics into the assembly line. This would enable quick feedback and modifications, which would further lower the chance of errors and increase operational effectiveness.

Case Study 2: Nutritional Production Efficiency Enhancement

DEFINE: What is the quality problem?

The main quality problem in nutritional production was found to be packing line inefficiencies, which resulted in considerable delays and more downtime. In addition to increasing operating expenses, this inefficiency also decreased output.

MEASURE: What are the measures?

The actions included monitoring packaging line production levels and downtime. As markers of line efficiency, specific measurements included the number of units packed each hour and the average downtime per shift.

ANALYSE: What is the root cause? How was it found?

Line inefficiencies were determined to be mostly caused by manual handling and machine setup delays, as demonstrated by time studies and workflow analysis. (Krotov et al., 2017)The examination of time logs and first-hand observations, which revealed bottlenecks in the manual portions of the packaging process, was the main method used to determine the fundamental causes.

IMPROVE: What was the solution?

The improvement entailed mechanising several crucial stages in the packing procedure to decrease dependence on human intervention. (Krotov et al., 2017)This involved upgrading the conveyor systems and installing automated cartooning equipment. To maximise their interactions with the new technology, staff members underwent retraining as well.

Discussion on Case's Accomplishments and Improvement Idea:

The packing process was automated, and after personnel training, line efficiency increased by 28%, surpassing the 25% goal. This raised the total output capacity while significantly lowering operating expenses. A further suggestion for enhancement would be to incorporate a real-time monitoring system to measure efficiency continually and promptly detect and resolve any new or reoccurring bottlenecks.

# Case Study 3: Steel Pipe Manufacturing Waste Reduction

DEFINE: What is the quality problem?

The production waste at the steel pipe manufacturing facility was a serious issue that affected environmental compliance and cost-effectiveness.

MEASURE: What are the measures?

The metrics were centred on how much waste material was produced and how well basic materials were utilized. (Saleh et al., 2023) The percentage of raw materials turned into scrap throughout the production process was one of the specific KPIs.

ANALYSE: What is the root cause? How was it found?

High scrap rates were mostly caused by antiquated equipment and ineffective material handling, according to analysis employing process mapping and waste audits. (Krotov et al., 2017)Data gathered on machine performance and scrap generation throughout various production shifts corroborated these conclusions.

IMPROVE: What was the solution?

Upgrading old equipment and putting in place a thorough scrap material recycling programme were the solutions. (Saleh et al., 2023) In order to guarantee the best possible handling and processing of materials, operator training was also improved.

Discussion on Case's Accomplishments and Improvement Idea:

The improvements and recycling programmes reduced trash by 23%, exceeding the 20% improvement objective. Significant financial savings and a smaller environmental effect were the outcomes of this. Adding to the efficiency and lowering waste might be achieved by putting in place an improved material requirement planning (MRP) system.

Synergy and Criticism

## Synergy:

The DMAIC technique demonstrates its resilience and flexibility in handling a range of industrial problems in each of the three case studies. The strategic emphasis on data-driven decision-making, process standardisation, and ongoing monitoring is a recurring motif in all of the applications. Every research project made use of these fundamental ideas to improve particular facets of their operations, proving that DMAIC is applicable to all industries. In order to decrease variability and boost efficiency, for example, the case studies on the automotive and nutritional industries both emphasised the need of implementing standardised procedures and automating operations. In a similar vein, the steel pipe manufacturing example reflected the same ideas of waste reduction and continual improvement through its emphasis on process efficiency and equipment upgrades. This synergy demonstrates how DMAIC may lead to significant operational gains in a variety of manufacturing settings.

## Critique and Improvements:

Even while each case study had some impressive successes, there are still some areas that may use improvement. Even though fault rates were much lower in the automobile assembly line scenario, the research might benefit from a deeper integration with technology, such as predictive analytics, to identify potential problems before they arise. By being proactive instead of reactive, this method may be able to prevent errors once they are discovered. The adoption of automated systems significantly increased productivity in the nutritional production case; however, a reliance on high-tech solutions might not address underlying systemic issues like process integration and employee training, indicating the need for a more comprehensive approach to improvement that takes human factors into account.

Finally, even if waste reduction was accomplished in the steel pipe manufacturing scenario, the main emphasis was on quick process improvements. To further strengthen their commitment to sustainability, they may investigate more creative recycling methods or implement more extensive environmental management systems in order to improve long-term sustainability. Every example shows that, even if DMAIC offers a robust framework for enhancement, deeper integration of novel technology and approaches is constantly possible to improve the long-term sustainability of process improvements.

# Conclusion

The application of the DMAIC methodology to three different studies in the automotive, chemical and metal industries provides concrete evidence of its effectiveness in solving industrial problems while improving efficiency and quality. This case study demonstrates the versatile and powerful nature of DMAIC as a problemsolving framework that not only adapts to a variety of industrial environments but also enhances.

By using the steps of Define, Measure, Analyse, Improve, and Control methodically, every industry was able to meet and frequently surpass its operating targets. By effectively lowering defect rates, the automobile sector improved consumer satisfaction and product quality. (Pugna et al., 2016)The nutritional industry improved productivity and cut expenses by making its packing lines more efficient. (Saleh et al., 2023) In the meantime, waste was drastically reduced in the steel pipe manufacturing sector, which helped to lower costs and maintain environmental sustainability. These accomplishments highlight DMAIC's ability to apply a methodical and systematic approach to produce quantifiable and significant outcomes.

Additionally, this analysis has shown that although DMAIC is very successful, its full potential can only be achieved in conjunction with ongoing observation and the incorporation of cutting-edge technology tools like real-time data tracking and predictive analytics. (Krotov et al., 2017)In order to guarantee that gains are long-lasting, these technologies can further boost the DMAIC phases' efficacy, especially in the Control phase.

In conclusion, the DMAIC technique may be a potent driver for change in manufacturing processes if it is implemented effectively. (Krotov et al., 2017) In addition to addressing current problems with efficiency and quality, it establishes the framework for further development. Adopting DMAIC offers a tried-and-true route to success for companies looking to attain operational excellence, highlighting the necessity of continual dedication to quality management procedures and the adoption of creative solutions to satisfy changing industry standards and customer expectations.

# References

- Pugna, A., Negrea, R., & Miclea, Ş. (2016). Using six Sigma methodology to improve the assembly process in an automotive company. Procedia - Social and Behavioral Sciences, 221, 308–316. https://doi.org/10.1016/j.sbspro.2016.05.120

- Saleh, B., El-Hadek, M. A., & Kouta, H. (2023). Waste reduction in pipe industry through lean six sigma implementation. The Journal of Solid Waste Technology and Management, 49(4), 408–421. https://doi.org/10.5276/jswtm/iswmaw/494/2023.408

- A six sigma approach towards improving quality management in manufacturing of nutritional products. (2017, June 1). IEEE Conference Publication | IEEE Xplore. https://ieeexplore.ieee.org/document/7985580